# Elephant YA jekketalje

Elephant jekketalje YA

- Jekketalje kapasitet 500-9000 kg

- Grade 105 galvanisert løftekjetting (DIN 5684)

- Taljene er pulverlakkert

- YA taljene er utstyrt med svivel kroker og sikkerhetslepper

- Lett å oppbevare i en verktøypose eller verktøykoffert

- Jekketaljen har lav vekt og er enkel å bruke

Dimensjoner YA (mm)

| Model | A (mm) | B (mm) | C (mm) | D (mm) | E (mm) | G (mm) | S (mm) | T (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| YA-50 | 112 | 91 | 239 | 178 | 69 | 24 | 36 | 13 |
| YA-80 | 144 | 122 | 290 | 269 | 91 | 23 | 36 | 15 |
| YA-100 | 144 | 122 | 312 | 269 | 91 | 28 | 43 | 16 |
| YA-160 | 162 | 137 | 353 | 310 | 99 | 29 | 43 | 21 |
| YA-320 | 186,5 | 180 | 411,5 | 310 | 104 | 36 | 53 | 28 |
| YA-630 | 186,5 | 236 | 564 | 310 | 104 | 47 | 70 | 34 |
| YA-900 | 186,5 | 300 | 688 | 310 | 104 | 73 | 85 | 47,5 |

Art. Y A 50

| Model |  | YA-50 |
| --- | --- | --- |
| Capacity ( ton ) |  | 0.5 |
| Standard lift ( m ) |  | 1.2 |
| Headroom ( mm ) |  | 240 |
| Hand effort to lift full working load ( kg ) |  | 37 |
| Load chain | Diameter ( mm ) | 4.3 |
|  | No. of falls | 1 |
|  | Grade of chain | V |
| Net weight ( kg ) |  | 2.8 |

Art. Y A 80

| Model |  | YA-80 |
| --- | --- | --- |
| Capacity ( ton ) |  | 0.8 |
| Standard lift ( m ) |  | 1.5 |
| Headroom ( mm ) |  | 290 |
| Hand effort to lift full working load ( kg ) |  | 30 |
| Load chain | Diameter ( mm ) | 5.6 |
|  | No. of falls | 1 |
|  | Grade of chain | V |
| Net weight ( kg ) |  | 6.0 |

Art. Y A 100

| Model |  | YA-100 |
| --- | --- | --- |
| Capacity ( ton ) |  | 1 |
| Standard lift ( m ) |  | 1.5 |
| Headroom ( mm ) |  | 312 |
| Hand effort to lift full working load ( kg ) |  | 37 |
| Load chain | Diameter ( mm ) | 5.6 |
|  | No. of falls | 1 |
|  | Grade of chain | V |
| Net weight ( kg ) |  | 6.2 |

Art. Y A 160

| Model |  | YA-160 |
| --- | --- | --- |
| Capacity ( ton ) |  | 1.6 |
| Standard lift ( m ) |  | 1.5 |
| Headroom ( mm ) |  | 352 |
| Hand effort to lift full working load ( kg ) |  | 36 |
| Load chain | Diameter ( mm ) | 7.1 |
|  | No. of falls | 1 |
|  | Grade of chain | V |
| Net weight ( kg ) |  | 9.2 |

Art. Y A 320

| Model |  | YA-320 |
| --- | --- | --- |
| Capacity ( ton ) |  | 3.2 |
| Standard lift ( m ) |  | 1.5 |
| Headroom ( mm ) |  | 420 |
| Hand effort to lift full working load ( kg ) |  | 44 |
| Load chain | Diameter ( mm ) | 9.0 |
|  | No. of falls | 1 |
|  | Grade of chain | V |
| Net weight ( kg ) |  | 15.5 |

Art. Y A 630

| Model |  | YA-630 |
| --- | --- | --- |
| Capacity ( ton ) |  | 6.3 |
| Standard lift ( m ) |  | 1.5 |
| Headroom ( mm ) |  | 564 |
| Hand effort to lift full working load ( kg ) |  | 45 |
| Load chain | Diameter ( mm ) | 9.0 |
|  | No. of falls | 2 |
|  | Grade of chain | V |
| Net weight ( kg ) |  | 26.5 |